Running code on the BeagleBone© Black

The purpose of this practical is to learn how to run OpenGL ES 2.0 code on the BeagleBone© Black embedded device and to manually install the PowerVR SDK.  Please note that a guide to using the prebuild PowerVR SDK is included in the teaching materials and is recommended over this manual installation.

It is vital that each step is following in the order presented and that each step is carefully completed.

The following are the main objectives:

- Learn how to install an operating system on a micro SD card

- Learn how to setup the BeagleBone© Black

- Learn how to communicate with the BeagleBone© Black

- Learn how to compile and run existing examples on the BeagleBone© Black

If you have a micro SC card that already contains a working operating system, then go to section 3.

# Download the Operating System

Go to https://rcn-ee.com/rootfs/bb.org/testing/ and select 2019-12-02 or you can use the latest date (at the time of writing the latest date was 2019-12-02 and this date is confirmed to work with the device).

Then select stretch-iot/ and you will be presented with something like the following in fig 1.

Fig 1 – example of operating systems

Then download the bone-debian operating system, in this example bone-debian-9.11-iot-armhf-2019-12-02-4gb.img.xz (see fig 1).

# Flashing the Operating System to a micro SD Card

Download and install balenaEtcher (https://www.balena.io/etcher/).

Use your computer's micro SD slot or a USB adapter to connect the micro SD card to your computer.  Use Etcher to write the operating system image to your micro SD card (see fig 2).

Fig 2 – Flashing the micro SD Card with an operating system image

# Booting from the micro SD Card

Insert the micro SD card into the BeagleBone© Black as indicated in fig 3.

Micro HDMI

Ethernet

USB to PC

Micro SD Card

SD Card Boot Button

Indicator LEDs

Fig 3. BeagleBone© Black

Insert an Ethernet cable as indicated in fig 3.  An internet connection is required to download some software.

Insert a micro HDMI cable as indicated in fig 3 and connect to a monitor or TV.  This will display the output of the device.

Connect a USB cable to your PC.  To boot the device from the micro SD card, you must hold down the SD Card Boot Button as indicated in fig 3, whilst you connect the USB cable to the device as indicated in fig 3.  This connection with provide power to the device and it will boot.  It is essential that you connect this cable last and that you are holding down the boot button as you connect it.  After the Indicator LEDs (as indicated in fig 3) are all turned on, you can release the Boot Button.

After the device has booted you will be presented with a screen as in fig 4.

Fig 4 – Boot successful

# Communicating with the BeagleBone© Black

If you have a USB keyboard, then you can connect it to the USB socket on the device. You can now go to section 5.

If you do not have a USB keyboard, then you need to install appropriate software to communicate with the BeagleBone© Black over the PC USB cable.  It is suggested that you install PuTTY (https://www.putty.org/) as it is used in this tutorial.

Find the COM port that the PC USB cable is using.  An easy way to find this is to open Device Manager and then open the Ports (COM & LPT) as shown in fig 5.

Fig 5 – USB COM Port

In this example the COM Port is 14.

Start PuTTY. Select Serial.  Enter your Port (in this example the COM Port is COM14). Enter the speed as 115200. You may want to save this information as a new Session (see fig 6).

Fig 6 - PuTTY

Then select Open.  You will then be presented with the login screen as in fig 4; if not then select Enter on your PC keyboard.

# Logging into the BeagleBone© Black

Enter both the username and password as root.  You will be presented by the prompt in fig 7.

Fig 7 – Logged in as root

# Installing the SDK and other Software onto the BeagleBone© Black

The following software and settings only need to be completed once. These changes will be made to the SD card image permanently and so these changes will remain the next time you boot the device.

The SD card image is 4GB but if you have a larger SD card then you can expand the available size of the SD card by running the following at the prompt.  Note, the black text is the prompt that will be displayed to you, and the red text is the command that you need to enter.

root@beaglebone:~# /opt/scripts/tools/grow_partition.sh

The next step is to add the ti-libgbm2 package as follows:

root@beaglebone:~# apt-get update

root@beaglebone:~# cd /opt

root@beaglebone:/opt# wget https://github.com/HunterEmbedded/sgx-bbb/raw/master/ti-sgx-ti33x-ddk-um-dev_1.14.3699939-git20171201.0-0rcnee9%7Estretch%2B20190328_armhf.deb

root@beaglebone:/opt# apt-get install ti-libgbm2-dev

The next step is to install cmake as follows:

root@beaglebone:/opt# apt-get install cmake

The next step is to install the PowerVR SDK and compile it as follows:

root@beaglebone:/opt# git clone --depth 1 https://github.com/powervr-graphics/Native_SDK.git

root@beaglebone:/opt# cd Native_SDK

root@beaglebone:/opt/Native_SDK# mkdir cmake-build

root@beaglebone:/opt/Native_SDK# cd cmake-build

root@beaglebone:/opt/Native_SDK/cmake-build# cmake .. -DPVR_BUILD_OPENGLES2_EXAMPLES=1 -DPVR_WINDOW_SYSTEM=NullWS -DCMAKE_BUILD_TYPE=Debug

root@beaglebone:/opt/Native_SDK/cmake-build# cmake --build .

Now the SDK and examples are built, run the OpenGLESIntroducingPVRUtils example as follows:

root@beaglebone:/opt/Native_SDK/cmake-build# cd bin

root@beaglebone:/opt/Native_SDK/cmake-build/bin# ./OpenGLESIntroducingPVRUtils

You will now see the output of the application on your monitor of TV connected by the micro HDMI cable.

# Installing your code onto the BeagleBone© Black

Installing your own code can be done by transferring files from one system to another, however the suggested method is to use your GitHub in the same way as it was used to install the PowerVR SDK.

You should create your own GitHub repository and upload the provided practical code.  The following example in fig 8 shows this in an example GitHub repository called Intro_Mobile_Graphics.

Fig 8 – example of a GitHub repository

This example repository can be installed on the BeagleBone© Black as follows (you need to change the GitHub link to your own repository):

root@beaglebone:/opt# cd /opt/Native_SDK/

root@beaglebone:/opt/Native_SDK# git clone --depth 1 https://github.com/darrenmckie/Intro_Mobile_Graphics.git

root@beaglebone:/opt/Native_SDK# cd Intro_Mobile_Graphics

root@beaglebone:/opt/Native_SDK/Intro_Mobile_Graphics# mkdir cmake-build

root@beaglebone:/opt/Native_SDK/Intro_Mobile_Graphics# cd cmake-build

root@beaglebone:/opt/Native_SDK/Intro_Mobile_Graphics/cmake-build# cmake .. -DPVR_WINDOW_SYSTEM=NullWS -DCMAKE_BUILD_TYPE=Debug

root@beaglebone:/opt/Native_SDK/Intro_Mobile_Graphics/cmake-build# cmake --build .

Now the Practical examples are built, run the IntroducingPVRUtils example as follows:

root@beaglebone:/opt/Native_SDK/Intro_Mobile_Graphics/cmake-build# cd bin

root@beaglebone:/opt/Native_SDK/Intro_Mobile_Graphics/cmake-build/bin# ./IntroducingPVRUtils

You will now see the output of the application on your monitor of TV connected by the micro HDMI cable.